The question we set out to answer

Why are crashes deadlier in some parts of Victoria than others (2012–2025)? Which mix of place, speed, road setting, and road-user type explains that—and what should we fix first?

How we worked (clear and simple)

We pulled the pieces of the same crash together using the accident number:

- accident.csv – when and what happened (deaths, speed zone, lighting, surface, road geometry).

- node.csv – where it happened (our four area types: Melbourne Urban, Rural Victoria, Small Cities, Towns).

- vehicle.csv – who was involved (flags for motorcycle-involved and heavy-vehicle-involved).

- road_surface_cond.csv and atmospheric_cond.csv – the road surface and weather at the time.

We compared five time windows: 2012–14, 2015–17, 2018–20, 2021–23, 2024–25.
For every slice we used one easy measure of severity: fatalities per 1,000 crashes.
We also grouped crashes by three practical lenses:

- Speed limit: ≤60, 70–80, ≥90 km/h.

- Road setting: between intersections (open road / “not at intersection”) vs at intersections (T or Cross).

- Road users: motorcycle-involved vs non-motorcycle; heavy-vehicle-involved vs other-vehicle.

For each group we always looked at how many crashes sit there (its share of crashes) and how deadly it is (fatalities per 1,000). For groups with very few crashes (for example, Urban ≥90), we kept them visible but treated the rate as not reliable.

What we found (same facts, explained more fully)

1) Baseline: the rural gap is real, persistent, and grows again late

Over the whole timeline, Rural Victoria is always the most deadly. Fatalities per 1,000 crashes rose from 11.85 → 16.06 in Urban areas and from 40.79 → 47.57 in Rural between 2012–14 and 2024–25. The gap tightened a little around 2018–20, then opened up again after 2021.
Plain meaning: we don’t just have one bad year; we have a stable pattern that needs explaining and fixing.

2) Speed: higher limits kill more, and Rural has more of that exposure

In 2024–25:

- Rural Victoria at ≥90 km/h sits around 86.45 per 1,000, vs 21.62 per 1,000 at ≤60 km/h. That’s about four times higher at the top speed band.

- Urban at ≤60 km/h is about 16.39 per 1,000; Urban ≥90 has very few crashes, so the rate is unstable/near zero and should be read with caution.

Across periods, Rural and Towns have many more crashes in the ≥90 band than Melbourne Urban.
Why this matters: rural areas spend more time in the most deadly speed band, and that band is much deadlier everywhere. Speed alone explains a large part of the rural severity premium.

3) Road setting: open road (mid-block) is where severity concentrates

In 2024–25 (Rural Victoria):

- “Not at intersection” (open road / mid-block) ≈ 57.22 per 1,000.

- T/Cross intersections ≈ 22.06–24.39 per 1,000.

Rural networks also have more of their crashes on open-road stretches than cities do.
Why this matters: the worst harm sits on mid-block high-speed corridors. When you combine riskier setting with more exposure, the gap grows.

4) Vehicle mix: motorcycles and heavy vehicles add extra risk in those same spots

Motorcycles (2024–25).

- Share of crashes: 22.33% Rural vs 10.79% Urban.

- Fatality rate: ~56.52 (Rural) vs 38.22 (Urban) per 1,000.
Where it’s ≥90 km/h and open road, motorcycle fatality rates are often several times higher than non-motorcycle.
Meaning: even at a modest share, motorcycles punch above their weight in deaths, especially on fast open roads.

Heavy vehicles (2024–25).

- Share of crashes is similar (3.79% Rural, 3.50% Urban).

- Fatality rate differs by context: ~51.28 Rural vs 58.82 Urban per 1,000, with the worst outcomes on mid-block rural links (passing, fatigue, and road standard issues).
Meaning: heavy-vehicle harm clusters on the same high-speed corridors, so fixing those corridors pays twice.

5) Lighting, surface, weather: important amplifiers, not the root cause

- Lighting: Dark–Unlighted is worse than daylight/lighted, but many rural deaths still occur in daylight because most rural driving is daytime on open roads.

- Surface: Loose/Gravel and Oil/Mud/Debris show higher fatality rates where they occur (often rural) but these buckets are small. Use them as red flags to investigate.

- Weather: a few wind/dust spikes appear in small numbers; most crashes—and deaths—happen in fine conditions.
Meaning: these factors raise risk at the edges; the core rural gap comes from speed + open road + who is on the road.

What it means (the story in one piece)

Rural Victoria’s higher death rate is not random. It comes mainly from more driving and crashing on high-speed, between-intersection roads, plus a road-user mix (motorcycles and heavy vehicles) that is more fragile in those same places. If we change that mix—manage speed and protect mid-block corridors, then target motorcycle and heavy-vehicle risks—the rural–urban severity gap will shrink.

What to do first (practical fixes that match the data)

1) High-speed open-road program (≥90 km/h).
Review limits on the worst mid-block links; add point-to-point cameras; use time-based lower limits at dusk/dawn and holiday peaks. Make roads more forgiving: median/edge barriers, sealed & wider shoulders, rumble strips, clear zones, better curve warnings and bright, reflective line-marking.

2) Stop run-off-road deaths.
Lay high-friction surfaces on bends and known blackspots; upgrade guardrails; remove or protect roadside hazards so a mistake isn’t fatal.

3) Motorcycle route packages.
On known rural riding routes: improve bend friction; fit motorcycle-friendly barriers; run speed/gear campaigns in peak seasons.

4) Heavy-vehicle corridor safety.
More rest bays and fatigue checks; add overtaking lanes/turnouts; widen lanes/shoulders where possible; review limits on steep or narrow sections.

5) Visibility where it counts.
Light rural nodes and town approaches; keep markers and signs bright; add dust/wind warnings on known trouble spots.

6) Keep surfaces safe.
Seal shoulders; clean oil/mud/debris quickly; where gravel persists, seal it or manage speeds until you can.

7) Keep score.
Flag very small groups so they don’t mislead; reduce “Unknown” fields; re-run these same charts each quarter to check the ≥90 open-road program is pushing fatality rates down in Rural and Towns.

Minimum figure set to tell this story (keep it tight)

- Baseline: fatal/1,000 by period × Geo4
insight_baseline_geo4.png

- Speed: severity + exposure
insight_speed_severity_all_periods_geo4.png
insight_speed_exposure_vs_severity_scatter_geo4.png

- Geometry: severity + exposure
insight_geometry_severity_all_periods_geo4.png
insight_geometry_exposure_vs_severity_scatter_geo4.png

- Motorcycles: share vs fatal rate (group-level)
mc_share_vs_fatal_rate_geo4.png

- Heavy vehicles: severity by geometry (with share labels)
hv_geometry_fatal_with_share_geo4.png

(Everything else can sit in an appendix for reviewers who want more detail.)

Our answer, after all the evidence

Crashes are deadliest where Rural Victoria spends more of its driving: high-speed, between-intersection roads. Add in motorcycles and heavy vehicles on those links and the death rate climbs further. The best fixes start there—manage speed and protect those corridors—then target motorcycle and heavy-vehicle risks, with selective lighting and steady surface upkeep.